COMS 4713 – M01

Fall 2020

Video Store USA

Paul Mickey

Contents

Introduction	4

Server Room	4

Security System	4

Warehouse Systems	4

Office Systems	5

Building Layout	6

End User Devices	9

Main Office and Warehouse	9

Individual Store	10

Management of Devices	12

Segment of Vlans	12

Cost Analysis	12

Servers	12

Main Office and Warehouse	12

Individual Store	13

Identity Management	14

Cloud Services	14

Cost Analysis	15

Communication Media	15

Cabling	15

Wireless	15

Main Office and Warehouse	15

Individual Store	16

Internet Provider	17

Main Office and Warehouse	17

Individual Store	17

On the Go	17

Network Configuration	17

Routers	17

Main Office and Warehouse	17

Individual Store	18

Switches	18

Main Office and Warehouse	18

Individual Store	18

Data Room	18

Main Office and Warehouse	18

Individual Store	18

Network Configuration	18

Main Office	19

Individual Store	19

Cost Analysis	20

Security	20

Anti-Virus Software	20

VPN Access	20

Firewall	20

Training	20

Backups	21

Other Network Communication	21

Digital Phone System	21

Main Office and Warehouse	21

Individual Store	21

Printers	21

Main Office and Warehouse	21

Individual Store	21

Other Devices	21

Main Office and Warehouse	21

Continuing IT Support	21

Conclusion	22

# Introduction

A growing regional video rental store called Video Store USA is building a new head office in Fort Smith, Arkansas to support their 150 stores that are located in AR, OK, TX, MO, and KS. This office will house around 50 employees to start with and will need to be planned out to eventually accommodate around 100 employees in the next 5 years or so. The primary functions of this office is to run administrative operations (things like accounting, district and regional management, among other admin tasks), be the main warehouse and distribution center for all the stores, and host the servers for both the stores and main office and warehouse tasks.

Our focus will be the networked systems that are housed in the main office and warehouse. This will consist of the server room systems, the security system throughout the building, the warehouse systems, and the systems in the office that are employee use.

# Server Room

The server room will serve as the central hub for the office and warehouse systems at the main office along with the computer systems at the stores which we will connect the stores to the main office using a site to site VPN and/or a leased line. The servers in the server room will be doing tasks like file storage, domain control, phone system, among other things. Things like web hosting, email hosting, and the main Point of Sale (PoS) database live in the cloud offsite. Also this room will contain the main switches, routers, etc… for the computer network.

# Security System

This will be in in the Loss Prevention and Security Office. This will solely focus on monitoring the security for the office and warehouse (store security systems will be onsite at each store). This will contain Power over Ethernet (PoE) switches (which will power IP cameras), video recorders for the cameras (also called a Network Video Recorder (NVR)), a basic Network Attached Storage (NAS) for saving video clips along with other documents for Loss Prevention, a networked printer, and other gear from the security company. The primary goal is for this office to provide security to the office and warehouse along with having a Loss Prevention office so everything is accounted for in the warehouse and stores.

There will be around 5 employees in this office so there will be the need for induvial workstations for them too (even if they work different shifts). These employees will also have an employee cell phone which will replace the need for each to have an IP phone (there will be a centralized IP phone in the office though)

# Warehouse Systems

This will be where sorting and distribution for the stores will take place. The big thing for this part of the office is to have enough wifi coverage for the portable scanners and portable printers. There will also be a few workstations with full-size networked printers scattered about the warehouse which will be primarily for bulk actions that cannot be done on a handheld (things like printing forms, checking email, printing labels in bulk, among other things).

The workflow is that the warehouse employees can receive deliveries (which will either contain DVDs, Blue-rays, CDs, video games, a limited number of video players that will be for sale at the stores, and promo material for the stores), log the deliveries into inventory via the scanners and/or workstations, sort through the inventory, add inventory labels to the inventory where needed, and either the inventory goes into storage (which has its own way of being sorted) or the inventory ready to ship out the next day via third party freight carrier (which will all be logged in the system). Stores may return inventory at times which will go through a similar process and will either go into storage or be sold online. Other items like candy, popcorn, and gift cards that are sold in-store are handled by a third-party vendor, so we are not concerned about that.

There will be around 10-15 employees in the warehouse throughout the day working different shifts (1st, 2nd, and 3rd) so the devices can be shared. IP phones will most than likely be wireless (which might be a dedicated device or the handheld scanners may contain that functionality). There will still be a central IP phone in the warehouse at one of the workstations.

# Office Systems

This will be the home of all the office employees. The departments are Senior Management (people like the owner, VPs, etc…), Junior Management (essentially made up of district executives which oversee a group of stores and work remotely quite a bit), Loss Prevention/Security (which is covered in another section), Accounting, Human Resources, IT, and a general administrative department.

Generally each employee will have a desk consisting of a computer (could be a laptop or desktop depending on the job) along with a second monitor, a IP phone, a company tablet (if an employee has a laptop they may not have one), (depending on the job) a company phone, and anything else an employee needs for their job.

Some items like printers and laptops might be shared among an department depending on the needs of that department. For example, accounting has an office made up of cubicles and they do not travel much, then they will have a centralized printer in their office and laptops that can be checked out if an employee needed to work outside of the office like at a conference.

Then in some departments like Junior Management which primarily contains district executives, will need their work environment to be mobile. They will have a laptop, a cell phone (which has their IP phone forwarded to), and possibly a company tablet. Their desks might be “hot-desks” which means they are shared depending on who is in the office. If it is a time of year where most district executives are in town, temporary workspaces may have to be setup in a conference room and compromises may need to be made (for example they may not have an IP phone or a docking station with a 2nd monitor, keyboard, and mouse). It will be important that we have enough Wifi and/or Ethernet drops in certain places.

Also, if a person is in Senior Management, they may have more devices than some employees. These employees most than likely have a private office, a desktop, a laptop, an IP phone, a dedicated printer, a tablet, and other items to make their job easier.

Finally, there will be other devices on the network like conference room video call systems, break room TV, guest Wi-Fi, among other things. Overall, there will be around 30-40 employees in this category.

# Building Layout

This will be a three-story building. The warehouse will be 10,000 sq ft while the office is around 15, 000 sq ft. Here are some rough diagrams (I was having issues with Visio; I plan to have more detailed drawings as time goes on).

Figure 1: 1st Floor Office and Warehouse

Figure 2: 2nd Floor Office and Warehouse

Figure 3: 3rd Floor Office

# End User Devices

For personal devices (both employees and guests), they will be allowed on the guest wifi and Vlan only and will not be allowed on the main network nor should they be used for work at all. If an employee is caught using their personal device for work reasons they could get fired since any work info is technically confidential.

Wired in devices will consist of things like phones, printers, desktop computers, TVs, etc… This is so we can cut down on wifi usage where needed to provide a better connection for everyone.

Wireless devices will consist of things like company issued cell phones, tablets, and laptops. For the warehouse employees they will have wireless IP phones and/or handheld scanners.

## Main Office and Warehouse

Devices that are company issued to main office and warehouse employees are as follows. Depending on the employees’ job functions will depend on what specific set of devices they will end up with. Generally, each department should be getting similar (in not the same) devices to maintain standardization.

| Device | Device Type | Connection Type | Unit Price | Quantity and location | Price |
| --- | --- | --- | --- | --- | --- |
| Cisco 8841 Desktop IP Phone | Office IP Phone | Wired | $135 | 1 Warehouse + 45 Office = 46 | $6,210 |
| Cisco 8821 Wireless IP Phone | Warehouse IP Phone | Wireless | $192 | 2 Warehouse | $384 |
| iPhone 12 Mini 64GB | Mobile Phone | Wireless | $699 | 18 Office | $12, 582 |
| Pixel 5 128GB | Mobile Phone | Wireless | $699 | 10 Office | $6,990 |
| iPad Air (4th Gen with LTE) 64GB | Wireless Tablet | Wireless | $729 | 10 Office | $7,290 |
| iPad 10.2” (8th Gen Wi-Fi only) 32GB | Wireless Tablet | Wireless | $329 | 20 Office | $6,580 |
| Zebra TC75x | Mobile Barcode Scanner/Terminal | Wireless | $1,767.27 | 5 Warehouse | $8,836.35 |
| Zebra ZQ61 | Mobile Printer | USB and Bluetooth | $795.41 | 5 Warehouse | $3,977.05 |
| Standalone Time Clock | Time Clock | USB Flash | $33.59 | 1 Warehouse | $33.59 |
| Dell Latitude 7300 with 256GB SSD Storage, 8GB RAM, i5-8365U | Laptop | Wireless | $749.00 | 23 Office | $17,227 |
| Dell Vostro 3681 with 1TB HDD Storage, 8GB RAM, i5 10400 | Desktop | Wired | $499 | 2 Warehouse + 33 Office = 35 | $17,465 |
| Dell 22’ – P2219H | Display | Wired | $159.99 | 2 Warehouse + 96 Office= 98 | $15,679.02 |
| Dell Universal Dock – D6000 | Docking Station | Wired/Wireless | $194 | 15 Office | $2,910 |
|  |  |  |  | Total | $93,582 |

For the warehouse, employees will share two Dell Desktops with a single monitor, five Zebra Mobile Terminals with printers, a desk IP phone, and two Wireless IP phones. This is because there is 1st,, 2nd, and 3rd shift crews that have staggered workdays and it more sense cost wise to share devices in most cases. Since all the warehouse employees are hourly (as compared to salaried), they will also have a centralized time clock in the warehouse. There are around 16 people that work the warehouse.

General office employees that do not travel (Accounting, Human Resources, and general administrative department) will be issued a dell desktop with 2 monitors, a company tablet, and a desk IP phone. There are 30 employees in this category.

Junior management employees (which are mainly district executives and other store support executive employees that mainly work remotely) will have a laptop, and a company cell phone. They will also have access to a one of 15 hot desks which will be equipped with two monitors, a docking station with a generic keyboard and mouse, and a desk VoIP phone. There are 20 of employees in this category.

Senior management (employees like the owner, VPs, etc…) will be issued a will be issued a desktop with two monitors, a laptop, a company cell phone, and a desk VOIP phone. There are 3 employees in this category.

## Individual Store

Devices that are company issued to each store are as follows. Depending on the employees’ job functions will depend on what specific set of devices they will end up with. Generally, each store should be getting similar (in not the same) devices to maintain standardization depending on the year of deployment. The devices listed below are just for a typical store (not all stores).

| Device | Device Type | Connection Type | Unit Price | Quantity | Price |
| --- | --- | --- | --- | --- | --- |
| Standalone Time Clock | Time Clock | USB Flash | $33.59 | 1 | $33.59 |
| Dell Vostro 3681 with 1TB HDD Storage, 8GB RAM, i5 10400 | Desktop | Wired | $499 | 2 | $998.00 |
| Dell 22’ – P2219H | Display | Wired | $159.99 | 3 | $479.97 |
| iPad Air (4th Gen with LTE) 64GB | Wireless Tablet | Wireless | $729 | 2 | $1,458.00 |
| iPad 10.2” (8th Gen Wi-Fi only) 32GB | Wireless Tablet | Wireless | $329 | 1 | $329.00 |
| Zebra TC75x | Mobile Barcode Scanner/Terminal | Wireless | $1,767.27 | 2 | $3,534.54 |
| Zebra ZQ61 | Mobile Printer | USB and Bluetooth | $795.41 | 2 | $1,590.82 |
| Epos Complete POS System | Point of Sale | Wired | $349 | 2 | $698.00 |
| Generic VoIP Phone | Store IP Phone | Wired | $100 | 3 | $300 |
|  |  |  |  | Total | $9,421.92 |

Each store will have a manager’s office that the Store Manager and Assistant Store Manager will use. In that office there will be a desktop computer with two monitors, and a VoIP phone managed by a third-party provider which I just ball parked on the price for that. Both the Store Manager and Assistant Store Manager will have company provided tablets issued to them. Typically for a regular store there will also be around 15 employees total (not including management).

In the backroom (which contains storage and receiving) there will be a desktop computer with one monitor, two handheld scanners, two label printers, an electronic time clock, and a VOIP phone managed by a third-party provider.

At the front of the store there will be two checkout lanes each equipped with a Point of Sale System equipped with the appropriate software and hardware needed. Shared with the two checkouts will be a tablet and a VOIP phone managed by a third-party provider.

## Management of Devices

In order to effectively manage each device, there will be a MDM (Mobile Device Manager) by Microsoft Intune in place for mobile devices and more traditional devices will be controlled by a Active Directory Domain Controller. Each device on average will be replaced on a 2-3 year lifecycle (or when needed). Some devices like VoIP phones and desktop computers might be used longer meanwhile other devices such as tablets, laptops, and cell phones should be replaced sooner.

## Segment of Vlans

Our Vlans will be segmented by use case and location. More details can be found under the Network Configuration selection.

## Cost Analysis

As shown above the cost will be around $93,582 for the main office and warehouse for end user devices and $9,421.92 for each store.

# Servers

## Main Office and Warehouse

At the main office there will be the following servers as follows.

| Use Case | Model | OS | Specs | Unit Price | Quantity | Total Price |
| --- | --- | --- | --- | --- | --- | --- |
| Domain Controller | PowerEdge R340 | Windows 2019 Standard | Xeon® E-2246G, 16GB DDR4 ECC, H330 RAID Controller, 4x 1TB 7.2K HDD, 2x M.2 240GB SSD, Dual, Hot-plug, Redundant Power Supply, 350W | $2,721.26 | 2 | $5,442.52 |
| File Storage | PowerVault MD1420 | Ubuntu Server | 12x 1.2TB 10K RPM SAS 12Gbps 2.5in Hot-plug Hard Drive, Rack Rails | $9,624.53 | 10 | $96,245.30 |
|  |  |  |  |  | Total | $101,687.82 |

For identity management each user will sign in using their username and password which is stored on the domain controller and Office Online. For storage each user will have a set amount of storage on a network drive along with a shared department drive of storage. In addition, each user will have a Outlook Online account with 1 TB of OneDrive Space. This will allow flexibity where needed.

## Individual Store

At each store there will be the following servers as follows.

| Use Case | Model | OS | Specs | Unit Price | Quantity | Total Price |
| --- | --- | --- | --- | --- | --- | --- |
| Domain Controller | PowerEdge R340 | Windows 2019 Standard | Xeon® E-2246G, 16GB DDR4 ECC, H330 RAID Controller, 4x 1TB 7.2K HDD, 2x M.2 240GB SSD, Dual, Hot-plug, Redundant Power Supply, 350W | $2,721.26 | 2 | $5,442.52 |
| File Storage | PowerVault MD1400 | Ubuntu Server | 4x 1.2TB 10K RPM SAS 12Gbps 2.5in Hot-plug Hard Drive,3.5in HYB CARR, Rack Rails | $5,471.79 | 2 | $10,943.58 |
|  |  |  |  |  | Total | $16,386.10 |

## Identity Management

For identity management each user will sign in using their username and password which is stored on the domain controller and Office Online. As for storage, regular store employees will have access an Outlook Online account with a 1TB of OneDrive storage, but they will not have access to regular office applications. For the Store Manager and Assistant Store Manager each of them will have a Outlook Online account with a 1TB of OneDrive storage and the full office suite, but no personal local network storage because it is simply not needed. Each store will however have a local server with shared storage which will be stored on the local server and a backup server in the office.

## Cloud Services

A big part of the company’s data lives in the cloud including Mail and Cloud Storage via Microsoft 365 and the main Inventory Database and Web Host via Linode. For Microsoft 365 all main office and warehouse employees and regular store management receive a Business Standard license meanwhile regular store employees receive a Business Basic license to save a little bit of money. As for Linode, the cost is estimated and could be less or more each month. The benefits of everything becoming centralized off site, it allows someone else to manage the hardware. Also a benefit of Microsoft 365 is the Intranet and Teams Pages/Groups.

| Use Case | Service | Plan | Features | Unit Price | Quantity | Total Price |
| --- | --- | --- | --- | --- | --- | --- |
| Mail and Cloud Storage | Microsoft 365 | Business Standard | Online Storage (1 TB), 50 GB mailbox, Intranet and Team sites, and Office | $12.50/Month | 71 | $887.50/Month |
| Company Mail | Microsoft 365 | Business Basic | Online Storage (1 TB), Intranet and Team sites, and 50 GB mailbox | $5.00/Month | 15 | $75/Month |
| Database Storage | Linode | Object Storage | 10TB storage, 1TB Outbound Transfer | $200/Month | 2 | $400/Month |
| Web Hosting | Linode | Shared Instance | 32GB Ram, 8 CPU Cores, 640GB SSD, 16TB Transfer, 40 GB/s in, 7000 Mb/s out | $160/Month | 2 | $320/Month |
|  |  |  |  | Total | $1,682.50/Month | $20,190/year |

## Cost Analysis

Based on the cost analysis, the main office and warehouse is spending $101,687.82 on servers, each store around $16,386.10, and for monthly online storage around $1,682.50/month (that is also around $20,190/year).

# Communication Media

## Cabling

For cabling we will be using CAT 6e cabling for the most part so long as the length and speed requirements are met. There is no current plan to use fiber.

For MDF and IDFs locations see the “Data Room” section for more info.

## Wireless

### Main Office and Warehouse

For wireless coverage at the main office and warehouse we are using the following.

| Access Point Model | Unit Price | Quantity | Price |
| --- | --- | --- | --- |
| Ubiquiti UniFi UAP-AC-PRO | $149.00 | 18 | $2,682 |

Each Access Point will be using PoE (Power over Ethernet) via CAT6 to provide both power and data connections. They will extend the network’s Vlans over the wireless by using different SSIDS. In Packet Tracer the WiFi passwords are “ArkansasTech1”.

| Vlan Number | Vlan Name | SSID |
| --- | --- | --- |
| 10 | Employees | Video_Employee |
| 20 | IT | Video_IT (Hidden) |
| 30 | VoIP_HQ | Video_VOIP (Hidden) |
| 40 | Store_Employees | Video_S_EMP |
| 50 | Guest | Video_Guest |
| 60 | Security_HQ | NA |

On these Access Points we will be primarly using 5Ghz Wi-Fi if possible (2.4Ghz should be avoided) but if we need 2.4Ghz Wi-Fi we will provide a low powered and different channel SSID for those devices. The preferred alternative is to run extra ethernet (if supported) to that device if 5 Ghz is not supported. We will try to avoid interference by doing some network mapping, picking appropriate Wi-Fi channels based on that mapping, and look non Wi-Fi devices and their frequencies. I figured around 18 Access Points should be fine for the main office.

### Individual Store

For wireless coverage at each store we are using the following.

| Access Point Model | Unit Price | Quantity | Price |
| --- | --- | --- | --- |
| Ubiquiti UniFi UAP-AC-PRO | $149.00 | 3 | $447 |

Each Access Point will be using PoE (Power over Ethernet) via CAT6 to provide both power and data connections. They will extend the network’s Vlans over the wireless by using different SSIDS.

| Vlan Number | Vlan Name | SSID |
| --- | --- | --- |
| 10 | Employees | Video_USA_Private |
| 20 | VoIP | NA |
| 30 | PoS | NA |
| 40 | Guest | Video_Guest |
| 50 | Security_Store | Video_Security (Hidden) |
| 60 | IT | Video_IT_Store (hidden) |

On these Access Points we will be primarily using 5Ghz Wi-Fi if possible (2.4Ghz should be avoided) but if we need 2.4Ghz Wi-Fi we will provide a low powered and different channel SSID for those devices. The preferred alternative is to run extra ethernet (if supported) to that device if 5 Ghz is not supported. We will try to avoid interference by doing some network mapping, picking appropriate Wi-Fi channels based on that mapping, and look non-Wi-Fi devices and their frequencies. I figured around 3 Access Points should be fine for each store (two on the sales floor and one in the back).

## Internet Provider

### Main Office and Warehouse

At our main office and warehouse the office will be using Suddenlink as it’s ISP, we will assume that we are in a Gigabit market and we will get two connections of the 1 Gbps/50 Mbps for $414.90/each (which equals $829.80/month). This is not including a cost for things like a dedicated line between the office and the stores (I could not find exact pricing on that). The Demarc point will be right outside the server room in the main office.

### Individual Store

At each store will need a connection with decent speed. What ISP each store uses depends on location, but for simplicity sake the flagship store next store to the main office will be using Suddenlink and the 300/30 Mbps plan for $149.90/month. We will also be purchasing their Business Hosted Voice Business Seat plan for $34.95/month for the store’s phone line. The Demarc point will be on the back of the store near the back door.

### On the Go

Employees who have a company cell phone will be using Verizon as their phone provider and mobile ISP which contains the connection for their company phone and a mobile hotspot under the Business Unlimited Start plan which starts at around $40/line/mo and you add in the 28 phone lines from the office that is around $1,120/month.

Certain employees who have a company tablet may have one with mobile data from Verizon under the Business Unlimited Start Tablet plan depending on their job description and if they travel or work remotely semi regularly. This plan is around $40/line/mo and once you add in the 10 iPads with data that comes to around $400/month for the office. For each store management team, it will be around $80/month.

For the employees who do not have a company cell phone (or in some cases a company tablet with mobile data) typically, but are traveling on behalf of the company will be able to check out one of the spare cell phones that has active service on it on a first come first serve basis. If the amount of company cell phones with service that are available for checkout run out then this will need to be revisited and more employees will possibly need a company cell phone or a company tablet with mobile data.

# Network Configuration

## Routers

### Main Office and Warehouse

For the main office we will be using a Ubiquiti EdgeRouter 6P which will cost $259.00 and will be located in our network rack in the server room. The plan is to have our ISPs modem be connected to a leased line in order to connect to the stores (if not a security gateway or a whole network VPN may be needed).

### Individual Store

For each store we will be using a Ubiquiti EdgeRouter X which will cost $96.97 and will be located in our network rack located in the back of the store. We will also have spare ones at our main office if they are needed.

## Switches

### Main Office and Warehouse

For our switches the company will be using 5 of the Ubiquiti Networks EdgeSwitch 24 which costs around $420.00 and will total around $2,100 and give us around 120 Ethernet ports and will provide us PoE for the APs and then also for the IP phones at each desk.

### Individual Store

Each store will also have one Ubiquiti Networks EdgeSwitch 24 which costs around $420.00 and should be enough for each stores networking needs.

## Data Room

### Main Office and Warehouse

The data room will be located in a closet in the IT office. This will be where the MDF will be located, along the appropriate patch panels, and the main network rack. Then 2nd floor will have a IDF, a patch panel, and a switch located in the Security/Loss Prevention office. On the third floor it will be in the supply closet. All of these spaces will have dedicated breakers tied to a generator/battery backup and will have it’s dedicated AC vent and exhaust.

### Individual Store

Then in each store there will be a small networking closet in the back of the store that will have the MDF, the main rack, patch panels, etc… This will be cooled via a cooling vent and will have battery backups connected to their own circuit. Since each store is only one story and does not have many networking devices there is no need for an IDF.

## Network Configuration

Each device in both the office and store can connect to the servers located on Network 10.10.10.1/24 which represents “the cloud” (main website, mail server, database, and file storage). Typically these devices would be on a subdomain but I gave them each a domain name. The only thing these servers can do is serve up webpages (I tried to get the mail and FTP services working but could not figure it out on Packet Tracer). These servers have static IPs from 10.10.10.10-10.10.10.40 and they increment up by 0.0.0.10. The DNS server IP address is 8.8.8.8.

### Main Office

In the Packet Tracer I have laid out the networks as follows (I could not get the VoIP Vlan working correctly for the phones). There is a server representing the IT Vlan, please note it is just there as a representation (no inter-routing Vlans or stuff like that on Packet Tracer).

| Vlan Number | Vlan Name | Usage | Network Address | Amount of Devices |
| --- | --- | --- | --- | --- |
| 10 | Employee_HQ | For Company Owned Computers and other Devices | 172.16.10.1/24 | 126 |
| 20 | IT | This is for Servers and such | 172.16.20.1/24 | 12 |
| 30 | VoIP_HQ | Phone (Could not get it to work on main network but could on a separate network) | 172.16.30.1/24 | 48 |
| 40 | Store_Employees | For when store employees visit or a IT employee needs to connect a VPN to a stores network and then to the main network which will allow them to isolate them from everyone else. | 172.16.40.1/24 | Varies (Mostly should be zero most of the time) |
| 50 | Guest_HQ | This is for Guests and for employees’ personal devices to connect to the internet. | 172.16.50.1/24 | Varies |
| 60 | Security_HQ | This is so the third-party security monitoring company can connect online. | 172.16.60.1/24 | 30 cameras |

### Individual Store

In the Packet Tracer I have laid out the networks as follows (I could not get the VoIP Vlan working correctly for the phones). There is a server representing the IT Vlan, please note it is just there as a representation (no inter-routing Vlans or stuff like that on Packet Tracer).

| Vlan Number | Vlan Name | Usage | Network Address | Amount of Devices |
| --- | --- | --- | --- | --- |
| 10 | Employee_Store | For Company Owned Computers and other Devices | 172.17.10.1/24 | 7 |
| 20 | Guest_Store | This is for customers’ and employees’ personal devices | 172.17.20.1/24 | Varies |
| 30 | VoIP_Store | Phone (Could not get it to work on main network but could on a separate network) | 172.17.30.1/24 | 3 |
| 40 | PoS | This is for the Point of Sale Systems in the front | 172.17.40.1/24 | 2 |
| 50 | Security_Store | This is so the third-party security monitoring company can connect online. | 172.17.50.1/24 | 30 cameras |
| 60 | IT_Store | This is for Servers and such | 172.17.60.1/24 | 4 |

## Cost Analysis

Overall networking wise this will cost around $5,041 for the main office and warehouse for networking equipment and each store will cost around $964. I would also leave a few thousand dollars aside for cabling around the office and stores too.

# Security

## Anti-Virus Software

For Anti-Virus I personally think Windows Defender should get the job done because it is built into Windows and is cost effective (free), so I do not think buying Anti-Virus software would make much sense.

## VPN Access

For VPN access, all main office employees (expect warehouse employees) and store management will have VPN access from anywhere. This was not shown in Packet Tracer because I could not figure out how to get a VPN to work properly (in real life I have setup Point to Point VPNs before). With that said Office 365 can be accessed outside the VPN which will be where most of the work happens anyways.

## Firewall

In order to do anything external to the main network you will need to VPN, because for security reasons the firewall does not have any open ports.

## Training

Users will have the opportunity to attend an IT training session in-person, online, or watch training videos uploaded to the company intranet.

## Backups

For backups, ideally the company would either want to pay a hosting provider to store their backups or have a Site to Site VPN setup to perform nightly backups so that in an event of a distaster you can recover. Software for Ubuntu would be things like Deja Dup, R Sync, and Timeshift which are all free. For Windows (both server and desktop) you can use Windows Backup for the Servers and then use OneDrive for the employee’s computer (if their OneDrive is full then use Windows Backup to a remote server). Those two options are already covered in the cost of other products. Personally, I would use a snapshotting backup to save space and allow file revisions to be recovered. If you need to recover from a disaster it is as simple as copying the data from the backup servers to something else and be going momentarily.

# Other Network Communication

## Digital Phone System

### Main Office and Warehouse

In packet tracer I show two phones on a separate network (192.168.10.1/24) talking to each other (extensions 54002 and 54001), but when I tried to replicate that on the HQ_Router’s network it would not work.

### Individual Store

For the store, the phone lines will be managed by Suddenlink.

## Printers

### Main Office and Warehouse

There will be at least one printer/copier on each floor that is attached to the network (not shown in packet tracer). The warehouse will have a separate printer/copier to print out full size paper if they cannot print out on their handheld printer. The printer I have selected is a Brother MFCL2720DW which costs around $230 and we will need 3 for the office and one for the warehouse which will total out to around $920. These can just be thrown on the main Vlan. I figured we are in the digital age so they will not be used a whole lot.

### Individual Store

Each store will have a printer connected to the office computer in the back. I selected this basic Canon TR8620 which costs around $180.

## Other Devices

### Main Office and Warehouse

Another thing that could be implanted is a conference room video call system (consist of a TV, Webcam, Phone, Microphone, Speakers, etc…), but that is outside of the scope for this project. Security also needs to be implanted, but I have skipped over it due to time constraints.

# Continuing IT Support

I would recommend keeping around 4-5 people on staff after the network is built to help prevent any future problems. This way you can always have a few people at the office supporting the network and a few out on the field where needed. I “future proofed” the network by using modern devices in my planning which will allow years of usage and I also wired up Cat6e cabling where needed. I negated risks from outside attackers by having the company data live in the cloud and locally by not having any open ports on the firewalls.

# Conclusion

This is a big project and should be treated as such. The final cost for gear for the Head Office and Warehouse and one store is as follows.

| Category | Price |
| --- | --- |
| End User Devices – Office/Warehouse | $93,582 |
| End User Devices - Store | $9,421.92 |
| Servers – Office/Warehouse | $101,687.82 |
| Servers - Store | $16,386.10 |
| APs- Office/Warehouse | $2,682.00 |
| APs- Store | $447.00 |
| Router – Office/Warehouse | $259.00 |
| Router - Store | $96.97 |
| Switches- Office/Warehouse | $2,100.00 |
| Switchers – Store | $420.00 |
| Cabling | $3,000.000 |
| Total | $230,083 |

Reoccurring monthly cost:

| Category | Price |
| --- | --- |
| Internet – Office/Warehouse | $829.80/month |
| Internet – Store | $149.90/month |
| Phone – Store | $34.95/month |
| Verizon – Cell | $1,120/month |
| Verizon – Tablet | $400/month |
| Hosting | $1,682.50/month |
| Total | $4,217.15/month |